Dynamic Routing Project

Cisco Packet Tracer Lab Documentation

RIP v2 Protocol Implementation

Three-Router Multi-LAN Network Topology

# 1. Project Overview

This documentation describes the implementation of a dynamic routing network using RIP version 2 (RIPv2) protocol in Cisco Packet Tracer. The project demonstrates how routers automatically learn and share network routing information without manual static route configuration.

The network consists of three routers interconnected via FastEthernet and Serial links, each serving a distinct LAN subnet, with end devices including PCs and servers distributed across the network.

## 1.1 Objectives

- Design and configure a multi-router network topology in Cisco Packet Tracer

- Assign appropriate IP addressing to all interfaces and end devices

- Configure RIPv2 dynamic routing protocol on all three routers

- Verify full network connectivity across all subnets

- Demonstrate cross-network web server access between distant hosts

## 1.2 Technologies Used

| Technology | Details |
| --- | --- |
| Simulation Tool | Cisco Packet Tracer (Student Edition) |
| Routing Protocol | RIP Version 2 (RIPv2) |
| Router Model | Cisco 1941 Series (3x) |
| Switch Model | Cisco 2960-24 Series (3x) |
| End Devices | 4x PC-PT, 2x Server-PT |
| Link Types | FastEthernet (LAN), Serial DCE (WAN) |

# 2. Network Topology

The network topology consists of three separate LANs connected through routers. Router0 and Router1 are connected via a FastEthernet link (12.12.12.0/24), while Router1 and Router2 are connected via a Serial link (24.24.24.0/24). Each router serves one LAN segment through a switch.

## 2.1 Topology Diagram Description

The logical topology is structured as follows:

- Router0 (left) — connects to Switch0 via Fa0/1 (LAN) and to Router1 via Fa0/0 (WAN link)

- Router1 (center) — connects to Switch1 via Fa0/1 (LAN), to Router0 via Fa0/0, and to Router2 via Se0/0/0

- Router2 (right) — connects to Switch2 via Fa0/1 (LAN) and to Router1 via Se0/0/0

- Each switch has two end devices connected via FastEthernet

## 2.2 WAN Link Types

- Router0 to Router1: FastEthernet (dashed line) — 12.12.12.0/24

- Router1 to Router2: Serial DCE link (red line) — 24.24.24.0/24

# 3. IP Addressing Scheme

The network uses three private LAN subnets and two WAN subnets, all using /24 prefix length. The addressing follows the 192.168.x.0/24 convention for LANs and dedicated subnets for inter-router links.

## 3.1 Network Subnets Summary

| Subnet | Network Address | Purpose | Gateway |
| --- | --- | --- | --- |
| LAN 1 (Pink) | 192.168.1.0/24 | PC0, PC1 | 192.168.1.254 |
| LAN 2 (Green) | 192.168.2.0/24 | Server0, Server1 | 192.168.2.254 |
| LAN 3 (Blue) | 192.168.3.0/24 | PC2, PC3 | 192.168.3.254 |
| WAN 1 | 12.12.12.0/24 | Router0 — Router1 | N/A |
| WAN 2 | 24.24.24.0/24 | Router1 — Router2 | N/A |

## 3.2 Router Interface Addressing

| Device | Interface | IP Address | Subnet Mask | Connected To |
| --- | --- | --- | --- | --- |
| Router0 | Fa0/0 | 12.12.12.x | 255.255.255.0 | Router1 Fa0/0 |
| Router0 | Fa0/1 | 192.168.1.254 | 255.255.255.0 | Switch0 |
| Router1 | Fa0/0 | 12.12.12.x | 255.255.255.0 | Router0 Fa0/0 |
| Router1 | Fa0/1 | 192.168.2.254 | 255.255.255.0 | Switch1 |
| Router1 | Se0/0/0 | 24.24.24.1 | 255.255.255.0 | Router2 Se0/0/0 |
| Router2 | Se0/0/0 | 24.24.24.x | 255.255.255.0 | Router1 Se0/0/0 |
| Router2 | Fa0/1 | 192.168.3.254 | 255.255.255.0 | Switch2 |

# 4. RIPv2 Configuration

RIP Version 2 (RIPv2) was configured on all three routers to enable dynamic route exchange. RIPv2 supports classless routing (CIDR), subnet masks, and includes the 'no auto-summary' command to prevent automatic route summarization.

## 4.1 Configuration Steps (Applied to Each Router)

The following commands were applied on each router. Below is an example from Router0 (shown as Router_1# in the CLI):

Router_1# config t

Router_1(config)# router rip

Router_1(config-router)# ver 2

Router_1(config-router)# no auto-summary

Router_1(config-router)# network 12.12.12.0

Router_1(config-router)# network 192.168.1.0

Router_1(config-router)# exit

Router_1(config)# exit

Router_1# wr

## 4.2 RIPv2 Network Statements Per Router

| Router | Network Statement 1 | Network Statement 2 |
| --- | --- | --- |
| Router0 | network 12.12.12.0 | network 192.168.1.0 |
| Router1 | network 12.12.12.0 | network 192.168.2.0 |
| Router1 (WAN) | network 24.24.24.0 | (in addition to above) |
| Router2 | network 24.24.24.0 | network 192.168.3.0 |

## 4.3 Key Configuration Options

- ver 2 — Enables RIPv2 (supports classless routing with subnet masks)

- no auto-summary — Prevents automatic summarization to classful boundaries

- network <address> — Advertises directly connected network into RIP process

- wr — Saves configuration to NVRAM (equivalent to write memory)

# 5. Verification & Testing

## 5.1 Initial State — Router0 (Before RIP)

Before configuring RIPv2, the show ip route command on Router0 showed only directly connected (C) routes:

12.0.0.0/24 is subnetted, 1 subnets

C    12.12.12.0 is directly connected, FastEthernet0/0

C    192.168.1.0/24 is directly connected, FastEthernet0/1

Notice: No routes to 192.168.2.0 or 192.168.3.0 were present, confirming that routing had not yet been configured.

## 5.2 Post-RIP State — Router2 Routing Table

After configuring RIPv2 on all three routers, Router2's routing table showed dynamically learned routes (marked R):

R    12.12.12.0 [120/2] via 24.24.24.1, 00:00:03, Serial0/0/0

C    24.24.24.0 is directly connected, Serial0/0/0

R    192.168.1.0/24 [120/2] via 24.24.24.1, 00:00:03, Serial0/0/0

R    192.168.2.0/24 [120/1] via 24.24.24.1, 00:00:03, Serial0/0/0

C    192.168.3.0/24 is directly connected, FastEthernet0/1

This confirms that Router2 learned about all three LAN networks and the WAN subnet through RIPv2 advertisements.

## 5.3 Routing Table Analysis

| Code | Network | Metric | Meaning |
| --- | --- | --- | --- |
| C | 24.24.24.0/24 | — | Directly connected to Router2 Serial interface |
| C | 192.168.3.0/24 | — | Directly connected to Router2 LAN interface |
| R | 192.168.2.0/24 | [120/1] | Learned via RIP, 1 hop away (Router1) |
| R | 192.168.1.0/24 | [120/2] | Learned via RIP, 2 hops away (Router1 → Router0) |
| R | 12.12.12.0/24 | [120/2] | WAN subnet learned via RIP, 2 hops away |

## 5.4 End-to-End Connectivity Test

The final test was performed from PC0 (192.168.1.x) in LAN1 accessing the web server at 192.168.2.1 (Server0) in LAN2 using PC0's Web Browser:

- Source: PC0 in LAN 1 (192.168.1.0/24)

- Destination: http://192.168.2.1 (Server0 in LAN 2)

- Result: SUCCESS — Cisco Packet Tracer default webpage loaded

This confirmed that RIPv2 was correctly propagating routes across all three routers, enabling full cross-LAN connectivity.

# 6. Dynamic vs. Static Routing Comparison

| Aspect | Static Routing | Dynamic Routing (RIPv2) |
| --- | --- | --- |
| Configuration | Manual per-router route entry | Automatic via protocol |
| Scalability | Poor — grows with network size | Good — self-adjusting |
| Fault Tolerance | None — manual update required | Automatic re-routing on failure |
| Updates | No — manual changes only | Yes — every 30 seconds (RIP) |
| Overhead | None | Small (RIP broadcasts) |
| Best For | Small, stable networks | Medium networks, dynamic topologies |

# 7. Conclusion

This project successfully demonstrated the implementation of RIPv2 dynamic routing across a three-router, three-LAN network topology in Cisco Packet Tracer. The key accomplishments include:

- Designed and connected a multi-router topology with both FastEthernet and Serial WAN links

- Assigned correct IP addressing to all router interfaces, switches, PCs, and servers

- Configured RIPv2 with no auto-summary on all three routers

- Verified automatic route propagation — Router2 learned all remote subnets dynamically

- Confirmed end-to-end connectivity with a successful HTTP web request from PC0 to Server0 across two router hops

The project illustrates the advantage of dynamic routing: once RIPv2 was enabled, all routes were learned automatically within seconds, eliminating the need for manual static route configuration on every router.

End of Documentation